SINGLE ZONE : JUMPER SETTINGS

Control PanelValueJumperEOLAlarmEOLAlarmHoneywell

(Ademco/Microtech)1k1kA1Cooper (Scantronic, Menvier) Texecom, Pyronix, Castle2k24k7BC2Aritech4k74k7C2Guardtec4k76k8C3Guardall4k14k1B2 & 4DSC5k65k6D3 & 4Europlex2k22k2BC5

Wiring Two PA2EWR Contacts in Series (Shared Zone)

Select the appropriate EOL and Alarm links for contact 1. Do not fit any links for contact 2. Link the Multi-contact jumper W to Y for contact 1 and Z to X for contact 2.

Contact 1

Fit EOL jumper.

Fit ALARM jumper.

Change multiple contact jumper as shown.

Contact 2

Do NOT fit EOL jumper.

Do NOT fit ALARM jumper.

Change multiple connector jumper as shown.

Zone wire

4

34

2

1

Zone wire

JP3

Tamper

JP3

Tamper

Alarm

W

Y

X

Z

Alarm

W

Y

X

Z

4

34

2

1

Double Push Panic Button

Part Code : PA2EWR

Security Grade 3 Environmental Class 2

Manufactured to BS EN50131-2-6

Knight Fire & Security Products Ltd

Unit 84, Seawall Road Industrial Estate

Tremorfa, Cardiff CF24 5TH

Tel:029 2048 8129

Fax:029 2048 9132

www.knightfireandsecurity.com

Operation:- Push Side buttons inwards. The microswitch will only operate when both buttons are pushed simultaneously.

Reset :- by using the reset key (supplied)

Wiring

Traditional (4 Wire) – Ensure no jumpers are fitted.  Use all four terminals as marked.

Supervised (EOL) – Fit the appropriate jumpers for the control panel (See Table).  Connect the loop wires to the terminals marked . Fit Jumpers to the multi-contact link W to X and Y to Z.

Other – Some applications may not require both resistors.  In these cases fit jumper(s) in EOL or Alarm position as appropriate

SINGLE ZONE : JUMPER SETTINGS

Control PanelValueJumperEOLAlarmEOLAlarmHoneywell

(Ademco/Microtech)1k1kA1Cooper (Scantronic, Menvier) Texecom, Pyronix, Castle2k24k7BC2Aritech4k74k7C2Guardtec4k76k8C3Guardall4k14k1B2 & 4DSC5k65k6D3 & 4Europlex2k22k2BC5

Wiring Two PA2EWR Contacts in Series (Shared Zone)

Select the appropriate EOL and Alarm links for contact 1. Do not fit any links for contact 2. Link the Multi-contact jumper W to Y for contact 1 and Z to X for contact 2.

Contact 1

Fit EOL jumper.

Fit ALARM jumper.

Change multiple contact jumper as shown.

Contact 2

Do NOT fit EOL jumper.

Do NOT fit ALARM jumper.

Change multiple connector jumper as shown.

Zone wire

4

34

2

1

Zone wire

JP3

Tamper

JP3

Tamper

Alarm

W

Y

X

Z

Alarm

W

Y

X

Z

4

34

2

1

Double Push Panic Button

Part Code : PA2EWR

Security Grade 3 Environmental Class 2

Manufactured to BS EN50131-2-6

Knight Fire & Security Products Ltd

Unit 84, Seawall Road Industrial Estate

Tremorfa, Cardiff CF24 5TH

Tel:029 2048 8129

Fax:029 2048 9132

www.knightfireandsecurity.com

Operation:- Push Side buttons inwards. The microswitch will only operate when both buttons are pushed simultaneously.

Reset :- by using the reset key (supplied)

Wiring

Traditional (4 Wire) – Ensure no jumpers are fitted.  Use all four terminals as marked.

Supervised (EOL) – Fit the appropriate jumpers for the control panel (See Table).  Connect the loop wires to the terminals marked . Fit Jumpers to the multi-contact link W to X and Y to Z.

Other – Some applications may not require both resistors.  In these cases fit jumper(s) in EOL or Alarm position as appropriate

SINGLE ZONE : JUMPER SETTINGS

Control PanelValueJumperEOLAlarmEOLAlarmHoneywell

(Ademco/Microtech)1k1kA1Cooper (Scantronic, Menvier) Texecom, Pyronix, Castle2k24k7BC2Aritech4k74k7C2Guardtec4k76k8C3Guardall4k14k1B2 & 4DSC5k65k6D3 & 4Europlex2k22k2BC5

Wiring Two PA2EWR Contacts in Series (Shared Zone)

Select the appropriate EOL and Alarm links for contact 1. Do not fit any links for contact 2. Link the Multi-contact jumper W to Y for contact 1 and Z to X for contact 2.

Contact 1

Fit EOL jumper.

Fit ALARM jumper.

Change multiple contact jumper as shown.

Contact 2

Do NOT fit EOL jumper.

Do NOT fit ALARM jumper.

Change multiple connector jumper as shown.

Zone wire

4

34

2

1

Zone wire

JP3

Tamper

JP3

Tamper

Alarm

W

Y

X

Z

Alarm

W

Y

X

Z

4

34

2

1

Double Push Panic Button

Part Code : PA2EWR

Security Grade 3 Environmental Class 2

Manufactured to BS EN50131-2-6

Knight Fire & Security Products Ltd

Unit 84, Seawall Road Industrial Estate

Tremorfa, Cardiff CF24 5TH

Tel:029 2048 8129

Fax:029 2048 9132

www.knightfireandsecurity.com

Operation:- Push Side buttons inwards. The microswitch will only operate when both buttons are pushed simultaneously.

Reset :- by using the reset key (supplied)

Wiring

Traditional (4 Wire) – Ensure no jumpers are fitted.  Use all four terminals as marked.

Supervised (EOL) – Fit the appropriate jumpers for the control panel (See Table).  Connect the loop wires to the terminals marked . Fit Jumpers to the multi-contact link W to X and Y to Z.

Other – Some applications may not require both resistors.  In these cases fit jumper(s) in EOL or Alarm position as appropriate

SINGLE ZONE : JUMPER SETTINGS

Control PanelValueJumperEOLAlarmEOLAlarmHoneywell

(Ademco/Microtech)1k1kA1Cooper (Scantronic, Menvier) Texecom, Pyronix, Castle2k24k7BC2Aritech4k74k7C2Guardtec4k76k8C3Guardall4k14k1B2 & 4DSC5k65k6D3 & 4Europlex2k22k2BC5

Wiring Two PA2EWR Contacts in Series (Shared Zone)

Select the appropriate EOL and Alarm links for contact 1. Do not fit any links for contact 2. Link the Multi-contact jumper W to Y for contact 1 and Z to X for contact 2.

Contact 1

Fit EOL jumper.

Fit ALARM jumper.

Change multiple contact jumper as shown.

Contact 2

Do NOT fit EOL jumper.

Do NOT fit ALARM jumper.

Change multiple connector jumper as shown.

Zone wire

4

34

2

1

Zone wire

JP3

Tamper

JP3

Tamper

Alarm

W

Y

X

Z

Alarm

W

Y

X

Z

4

34

2

1

Double Push Panic Button

Part Code : PA2EWR

Security Grade 3 Environmental Class 2

Manufactured to BS EN50131-2-6

Knight Fire & Security Products Ltd

Unit 84, Seawall Road Industrial Estate

Tremorfa, Cardiff CF24 5TH

Tel:029 2048 8129

Fax:029 2048 9132

www.knightfireandsecurity.com

Operation:- Push Side buttons inwards. The microswitch will only operate when both buttons are pushed simultaneously.

Reset :- by using the reset key (supplied)

Wiring

Traditional (4 Wire) – Ensure no jumpers are fitted.  Use all four terminals as marked.

Supervised (EOL) – Fit the appropriate jumpers for the control panel (See Table).  Connect the loop wires to the terminals marked . Fit Jumpers to the multi-contact link W to X and Y to Z.

Other – Some applications may not require both resistors.  In these cases fit jumper(s) in EOL or Alarm position as appropriate